MORSE-64

FORMAL SYSTEM SPECIFICATION

Prefix-Invariant Morse Transmission Core and Multi-Modal Projection Layer

| Specification | Version 1.0 |
| --- | --- |
| Architecture | CERBERUS Semantic Kernel / Bitlinking |
| Word Family | MORSE-32 -> MORSE-64 -> higher-resolution extensions |
| Date | 12 August 2026 |

REFERENCE PRINCIPLE MORSE-32 is the canonical transmitted-signal semantic core. MORSE-64 preserves MORSE-32 byte-for-byte as B000-B031 and appends a 32-bit projection layer that specifies how the same Morse signal is timed, rendered, performed, observed, or evaluated across audio, radio, light, haptic, mechanical, digital, spatial, and other modalities.

Status: Formal reference specification for implementation, testing, education, and evolution.

# Document Control

| Field | Specification |
| --- | --- |
| Document Title | MORSE-64 Formal System Specification |
| Specification Version | 1.0 |
| Primary Subject | Compact Morse signal encoding plus prefix-invariant projection semantics |
| Canonical Base Word | MORSE-32, 32 bits / 4 bytes |
| Projection Word | MORSE-64, 64 bits / 8 bytes |
| Reference Morse Standard | ITU-R M.1677-1, International Morse code |
| Normative Language | MUST, MUST NOT, SHOULD, SHOULD NOT, and MAY are used as requirements terms. |
| Related CERBERUS Specifications | Semantic Kernel Bitlinking; Semantic Prefix Invariance; CERBERUS Matryoshka Integrated Architecture |

# Abstract

MORSE-64 defines a compact, deterministic semantic word family for representing Morse signals and projecting them into physical or perceptual forms. The first 32 bits, MORSE-32, encode the transmitted Morse signal as a registered semantic token with an exact sentinel-based binary tree path, token identity, class, role, profile, and Kernel version. The appended 32 bits form the MORSE-64 Projection Layer: a bounded description of unit duration, time basis, timing profile, output medium, expression mode, intensity, and fidelity.

The architecture separates the invariant signal from the variable projection. The letter A remains the same MORSE-32 token whether it is heard as a tone, flashed as light, felt as a haptic pulse, keyed mechanically, represented geometrically, or rendered by a teaching interface. Bitlinks can therefore bind once to stable MORSE-32 semantic addresses while MORSE-64 chooses how that meaning is expressed in a particular environment.

CORE RESULT MORSE-64 = MORSE-32 transmitted-signal semantics + 32 bits of projection semantics. Higher-resolution descendants may append further bits without moving or reinterpreting any existing address.

# Contents

| Ref. | Section |
| --- | --- |
| 1 | Purpose and Scope |
| 2 | Normative Concepts and Terminology |
| 3 | Design Principles |
| 4 | Canonical Word Architecture |
| 5 | MORSE-32 Transmission Core |
| 6 | Canonical Symbol and Signal Registry |
| 7 | MORSE-64 Projection Layer |
| 8 | Projection Execution Model |
| 9 | Semantic Kernel Bitlinking |
| 10 | Semantic Prefix Invariance and Resolution Expansion |
| 11 | Worked Examples |
| 12 | Runtime, Serialization, and Storage |
| 13 | Conformance, Validation, and Testing |
| 14 | Governance, Profiles, and Extensions |
| 15 | Implementation Templates |
| 16 | Applications |
| 17 | Design Principles Summary |
| 18 | Conclusion |
| A | Appendix A - Complete Bit List |
| B | Appendix B - Binary Examples |
| C | Appendix C - Registered Values and Reserved Ranges |
| G | Glossary |
| I | Subject Index |
| R | References |

# 1  Purpose and Scope

This specification defines MORSE-32 and MORSE-64 as a prefix-invariant Semantic Kernel family. MORSE-32 encodes the atomic signal intended for transmission. MORSE-64 appends the information required to project that signal through a selected medium and timing/performance context. The design is intended to support ordinary text-to-Morse conversion, human keying, machine transmission, multimodal accessibility, education, simulation, Bitlink-driven interfaces, and later scientific-resolution extensions without requiring migration of existing bits.

## 1.1  In Scope

A 32-bit transmitted-signal core with a 9-bit sentinel-prefixed Morse tree code.

An 8-bit Token ID that distinguishes semantic tokens even when two tokens share the same physical dot/dash sequence.

Typed Symbol Class, Role, Profile, and Version fields.

A 32-bit projection extension containing timing, medium, expression, intensity, and fidelity semantics.

Canonical International Morse mappings for letters, figures, punctuation, miscellaneous signs, and procedural signals aligned to ITU-R M.1677-1.

Deterministic decoding, projection, Bitlinking, serialization, extension, and conformance behavior.

Prefix-preserving interoperability between MORSE-32, MORSE-64, and future larger words.

## 1.2  Out of Scope

Replacing the ITU definition of International Morse code or claiming this specification is an ITU standard.

Encoding an arbitrary audio waveform, RF waveform, raw sensor stream, or every measured timing sample directly inside 64 bits.

Treating case, font, handwriting, or source-document typography as part of the transmitted Morse signal unless carried by a separate text/source model.

Guessing the meaning of unresolved or malformed signals. Fail-closed behavior is preferred.

Forcing community, historical, experimental, or fictional Morse variants into the canonical ITU profile without explicit profile identity.

# 2  Normative Concepts and Terminology

| Term | Definition |
| --- | --- |
| MORSE-32 | The first 32 bits. Canonical transmitted-signal semantic core. |
| MORSE-64 | MORSE-32 plus a 32-bit Projection Layer. |
| SignalCode9 | Nine-bit field containing a left-zero-padded sentinel-prefixed dot/dash path. |
| Sentinel | Leading binary 1 placed before the dot/dash path so sequence length remains recoverable. |
| Dot / Dash | Dot maps to binary 0; dash maps to binary 1 after the sentinel. |
| Token ID | Eight-bit semantic identity that disambiguates tokens sharing a physical sequence. |
| Projection | A replaceable rendering or performance of the semantic signal through a medium. |
| Timing Profile | Registered Model describing how dot units map to dash and gap durations. |
| Medium | Audio, RF/carrier, light, haptic, mechanical, digital, spatial, or other renderer class. |
| Bitlink | Declared connection from a Kernel state, field, element, or transition to another semantic or perceptual consumer. |
| Semantic Prefix Invariance | Requirement that a published prefix remain byte-for-byte stable inside conforming higher-resolution descendants. |
| Fidelity | Compact normalized indication of signal/performance/interpretation quality; not a substitute for detailed measurement data. |

# 3  Design Principles

| Principle | Meaning |
| --- | --- |
| Signal Before Projection | The Morse token is authoritative; audio, light, haptic, geometry, and other outputs are projections of that token. |
| Exact Binary Path | The physical Morse sequence is recoverable directly from SignalCode9 without consulting a display string. |
| Semantic Disambiguation | Token ID and Symbol Class distinguish identical signal patterns used for different semantic purposes. |
| Minimum Sufficient Resolution | Use MORSE-32 when only the signal matters; use MORSE-64 when projection/performance matters; use larger words only when additional resolution is required. |
| Prefix Invariance | Higher-resolution words append; they do not migrate, renumber, or reinterpret prior bits. |
| One Signal, Many Modalities | Multiple renderers may subscribe to one MORSE-32 signal without creating competing truth stores. |
| State Small, Contract Rich | Keep live words compact and store detailed Models, Rules, profile definitions, and resource bindings in Registries. |
| AI Proposes, Runtime Governs | Natural-language or AI routing may select tokens and projection profiles, but deterministic Kernel contracts validate the packed word and rendering behavior. |

# 4  Canonical Word Architecture

## 4.1  MORSE-32 Core Layout

| Bits | Group | Width | Purpose |
| --- | --- | --- | --- |
| B000-B008 | SignalCode9 | 9 | Sentinel + dot/dash tree path; exact transmitted pattern. |
| B009-B016 | TokenID8 | 8 | Semantic token identity; 256 possible registered values. |
| B017-B020 | SymbolClass4 | 4 | Letter, figure, punctuation, procedural, extended, custom, etc. |
| B021-B023 | Role3 | 3 | Ordinary atomic token or stream/control role. |
| B024-B027 | Profile4 | 4 | Canonical ITU or explicitly registered extension/historical/custom profile. |
| B028-B031 | Version4 | 4 | MORSE-32 contract version. v1.0 uses 0001. |

Figure 1. MORSE-32 transmitted-signal layout

MORSE-32 (32 bits / 4 bytes) B000 B008 B009 B016 B017 B020 B021 B023 B024 B027 B028 B031 ¦ SignalCode9 ¦ TokenID8 ¦Class4¦ Role3 ¦Profile4¦Version4¦

## 4.2  MORSE-64 Extension Layout

| Bits | Group | Width | Purpose |
| --- | --- | --- | --- |
| B000-B031 | MORSE-32 | 32 | Inherited transmitted-signal semantic core. |
| B032-B041 | UnitDuration10 | 10 | Base dot-unit scalar, 0-1023. |
| B042-B043 | TimeBasis2 | 2 | Microseconds, milliseconds, 10 ms units, or external Model. |
| B044-B047 | TimingProfile4 | 4 | Registered timing/performance grammar. |
| B048-B051 | Medium4 | 4 | Projection medium / renderer class. |
| B052-B055 | ExpressionMode4 | 4 | Generated, keyed, observed, decoded, training, simulated, etc. |
| B056-B059 | Intensity4 | 4 | Normalized projection magnitude, 0-15. |
| B060-B063 | Fidelity4 | 4 | Normalized quality/confidence score, 0-15. |

Figure 2. Prefix-invariant MORSE-64 construction

MORSE-64 (64 bits / 8 bytes) [ B000 ---------------- B031 ][ B032 ------------------------------ B063 ] [ MORSE-32 ][ PROJECTION-32 ] MORSE-32 is never rewritten when projection detail is appended.

## 4.3  Complete Bit List

| Bit | Group | Definition |
| --- | --- | --- |
| B000 | SignalCode9 | SignalCode9 field bit 0 (MSB-first across B000-B008). |
| B001 | SignalCode9 | SignalCode9 field bit 1 (MSB-first across B000-B008). |
| B002 | SignalCode9 | SignalCode9 field bit 2 (MSB-first across B000-B008). |
| B003 | SignalCode9 | SignalCode9 field bit 3 (MSB-first across B000-B008). |
| B004 | SignalCode9 | SignalCode9 field bit 4 (MSB-first across B000-B008). |
| B005 | SignalCode9 | SignalCode9 field bit 5 (MSB-first across B000-B008). |
| B006 | SignalCode9 | SignalCode9 field bit 6 (MSB-first across B000-B008). |
| B007 | SignalCode9 | SignalCode9 field bit 7 (MSB-first across B000-B008). |
| B008 | SignalCode9 | SignalCode9 field bit 8 (MSB-first across B000-B008). |
| B009 | TokenID8 | TokenID8 field bit 0 (MSB-first across B009-B016). |
| B010 | TokenID8 | TokenID8 field bit 1 (MSB-first across B009-B016). |
| B011 | TokenID8 | TokenID8 field bit 2 (MSB-first across B009-B016). |
| B012 | TokenID8 | TokenID8 field bit 3 (MSB-first across B009-B016). |
| B013 | TokenID8 | TokenID8 field bit 4 (MSB-first across B009-B016). |
| B014 | TokenID8 | TokenID8 field bit 5 (MSB-first across B009-B016). |
| B015 | TokenID8 | TokenID8 field bit 6 (MSB-first across B009-B016). |
| B016 | TokenID8 | TokenID8 field bit 7 (MSB-first across B009-B016). |
| B017 | SymbolClass4 | SymbolClass4 field bit 0. |
| B018 | SymbolClass4 | SymbolClass4 field bit 1. |
| B019 | SymbolClass4 | SymbolClass4 field bit 2. |
| B020 | SymbolClass4 | SymbolClass4 field bit 3. |
| B021 | Role3 | Role3 field bit 0. |
| B022 | Role3 | Role3 field bit 1. |
| B023 | Role3 | Role3 field bit 2. |
| B024 | Profile4 | Profile4 field bit 0. |
| B025 | Profile4 | Profile4 field bit 1. |
| B026 | Profile4 | Profile4 field bit 2. |
| B027 | Profile4 | Profile4 field bit 3. |
| B028 | Version4 | Version4 field bit 0. |
| B029 | Version4 | Version4 field bit 1. |
| B030 | Version4 | Version4 field bit 2. |
| B031 | Version4 | Version4 field bit 3. |
| B032 | UnitDuration10 | UnitDuration10 field bit 0. |
| B033 | UnitDuration10 | UnitDuration10 field bit 1. |
| B034 | UnitDuration10 | UnitDuration10 field bit 2. |
| B035 | UnitDuration10 | UnitDuration10 field bit 3. |
| B036 | UnitDuration10 | UnitDuration10 field bit 4. |
| B037 | UnitDuration10 | UnitDuration10 field bit 5. |
| B038 | UnitDuration10 | UnitDuration10 field bit 6. |
| B039 | UnitDuration10 | UnitDuration10 field bit 7. |
| B040 | UnitDuration10 | UnitDuration10 field bit 8. |
| B041 | UnitDuration10 | UnitDuration10 field bit 9. |
| B042 | TimeBasis2 | TimeBasis2 field bit 0. |
| B043 | TimeBasis2 | TimeBasis2 field bit 1. |
| B044 | TimingProfile4 | TimingProfile4 field bit 0. |
| B045 | TimingProfile4 | TimingProfile4 field bit 1. |
| B046 | TimingProfile4 | TimingProfile4 field bit 2. |
| B047 | TimingProfile4 | TimingProfile4 field bit 3. |
| B048 | Medium4 | Medium4 field bit 0. |
| B049 | Medium4 | Medium4 field bit 1. |
| B050 | Medium4 | Medium4 field bit 2. |
| B051 | Medium4 | Medium4 field bit 3. |
| B052 | ExpressionMode4 | ExpressionMode4 field bit 0. |
| B053 | ExpressionMode4 | ExpressionMode4 field bit 1. |
| B054 | ExpressionMode4 | ExpressionMode4 field bit 2. |
| B055 | ExpressionMode4 | ExpressionMode4 field bit 3. |
| B056 | Intensity4 | Intensity4 field bit 0. |
| B057 | Intensity4 | Intensity4 field bit 1. |
| B058 | Intensity4 | Intensity4 field bit 2. |
| B059 | Intensity4 | Intensity4 field bit 3. |
| B060 | Fidelity4 | Fidelity4 field bit 0. |
| B061 | Fidelity4 | Fidelity4 field bit 1. |
| B062 | Fidelity4 | Fidelity4 field bit 2. |
| B063 | Fidelity4 | Fidelity4 field bit 3. |

# 5  MORSE-32 Transmission Core

## 5.1  SignalCode9: Sentinel-Based Binary Tree Encoding

SignalCode9 is the exact atomic transmitted pattern. It uses the natural binary structure of Morse: DOT = 0 and DASH = 1. A leading sentinel 1 is prepended before the Morse elements. The resulting bit string is then left-padded with zeros to exactly nine bits. The sentinel makes the path length recoverable even when the Morse sequence begins with one or more dots.

Figure 3. SignalCode9 encoding

Encoding rule 1. Start with sentinel: 1 2. Append one bit per Morse element: DOT = 0 DASH = 1 3. Left-pad with 0 until the field is 9 bits wide. Examples E . -> 10 -> 000000010 T - -> 11 -> 000000011 A .- -> 101 -> 000000101 N -. -> 110 -> 000000110 S ... -> 1000 -> 000001000 O --- -> 1111 -> 000001111 ERROR ........ -> 100000000 -> 100000000

Nine bits are sufficient for the atomic signal set in ITU-R M.1677-1 because the longest listed signal is the eight-dot Error signal. SignalCode9 = 000000000 is reserved for non-signal/control use and MUST NOT represent an ordinary atomic Morse signal.

## 5.2  Natural Binary Morse Tree

Figure 4. First levels of the Morse binary tree

1 ROOT / \ DOT 0 DASH 1 / \ 10 E 11 T / \ / \ 100 I 101 A 110 N 111 M / \ / \ / \ / \ 1000 S 1001 U 1010 R 1011 W 1100 D 1101 K 1110 G 1111 O The stored 9-bit field is the same tree address left-padded with zeros.

## 5.3  TokenID8: Exact Semantic Identity

SignalCode9 answers what physical dot/dash pattern is transmitted. TokenID8 answers what registered semantic token the sender or decoder means. This distinction is necessary because International Morse reuses some patterns for more than one semantic role. For example, K and the procedural Invitation to Transmit both use -.-; X and the multiplication sign both use -..-.

DISAMBIGUATION LAW Two MORSE-32 words MAY share SignalCode9 while remaining semantically distinct through TokenID8 and SymbolClass4. A decoder MUST NOT infer semantic equivalence solely from a shared dot/dash path.

## 5.4  SymbolClass4

| Value | Class |
| --- | --- |
| 0000 | Null / Unassigned |
| 0001 | Letter |
| 0010 | Figure |
| 0011 | Punctuation |
| 0100 | Miscellaneous sign |
| 0101 | Procedural signal |
| 0110 | Boundary / Control |
| 0111 | Composite source sign |
| 1000 | Extended alphabet |
| 1001 | Historical |
| 1010 | Custom / Community |
| 1011 | FASK / Fictional |
| 1100-1111 | Reserved |

## 5.5  Role3

| Value | Role |
| --- | --- |
| 000 | Ordinary atomic signal |
| 001 | Word boundary/control |
| 010 | Line boundary/control |
| 011 | Message start |
| 100 | Message end |
| 101 | Composite continuation / expansion context |
| 110 | Reserved |
| 111 | Custom registered role |

Role3 describes stream or operational use without changing the physical signal identity. A cross/addition signal may therefore retain its token while Role3 indicates that it is being used as a message-end marker in a particular workflow. Implementations SHOULD keep operational interpretation explicit rather than silently overloading punctuation semantics.

## 5.6  Profile4

| Value | Profile |
| --- | --- |
| 0000 | ITU International / M.1677-1 canonical profile |
| 0001 | Registered language extension |
| 0010 | Historical Morse profile |
| 0011 | Amateur / community registered profile |
| 0100 | Educational profile |
| 0101 | Experimental profile |
| 0110 | Custom registered profile |
| 0111 | FASK / fictional profile |
| 1000-1111 | Reserved for future profile families |

## 5.7  Version4

Version4 identifies the MORSE-32 semantic contract. This specification assigns 0001 to MORSE-32 v1.x. Prefix-preserving compatible clarifications MAY retain the same version where bit meanings do not change. Any change that reinterprets an existing published bit or value is non-conforming and requires a new Kernel identity or explicit translation rather than silent reuse.

# 6  Canonical Symbol and Signal Registry

The tables in this section align the canonical ITU profile with ITU-R M.1677-1. MORSE-64 is not an ITU Recommendation; it is a CERBERUS semantic encoding that uses the ITU signal mappings as the canonical v1 reference profile. Token IDs below are MORSE-32 registry assignments created by this specification.

## 6.1  Letters

| Symbol | Morse | SignalCode9 | Token ID | Token Binary |
| --- | --- | --- | --- | --- |
| A | .- | 000000101 | 0x01 | 00000001 |
| B | -... | 000011000 | 0x02 | 00000010 |
| C | -.-. | 000011010 | 0x03 | 00000011 |
| D | -.. | 000001100 | 0x04 | 00000100 |
| E | . | 000000010 | 0x05 | 00000101 |
| F | ..-. | 000010010 | 0x06 | 00000110 |
| G | --. | 000001110 | 0x07 | 00000111 |
| H | .... | 000010000 | 0x08 | 00001000 |
| I | .. | 000000100 | 0x09 | 00001001 |
| J | .--- | 000010111 | 0x0A | 00001010 |
| K | -.- | 000001101 | 0x0B | 00001011 |
| L | .-.. | 000010100 | 0x0C | 00001100 |
| M | -- | 000000111 | 0x0D | 00001101 |
| N | -. | 000000110 | 0x0E | 00001110 |
| O | --- | 000001111 | 0x0F | 00001111 |
| P | .--. | 000010110 | 0x10 | 00010000 |
| Q | --.- | 000011101 | 0x11 | 00010001 |
| R | .-. | 000001010 | 0x12 | 00010010 |
| S | ... | 000001000 | 0x13 | 00010011 |
| T | - | 000000011 | 0x14 | 00010100 |
| U | ..- | 000001001 | 0x15 | 00010101 |
| V | ...- | 000010001 | 0x16 | 00010110 |
| W | .-- | 000001011 | 0x17 | 00010111 |
| X | -..- | 000011001 | 0x18 | 00011000 |
| Y | -.-- | 000011011 | 0x19 | 00011001 |
| Z | --.. | 000011100 | 0x1A | 00011010 |
| É | ..-.. | 000100100 | 0x1B | 00011011 |

## 6.2  Figures

| Symbol | Morse | SignalCode9 | Token ID | Token Binary |
| --- | --- | --- | --- | --- |
| 0 | ----- | 000111111 | 0x20 | 00100000 |
| 1 | .---- | 000101111 | 0x21 | 00100001 |
| 2 | ..--- | 000100111 | 0x22 | 00100010 |
| 3 | ...-- | 000100011 | 0x23 | 00100011 |
| 4 | ....- | 000100001 | 0x24 | 00100100 |
| 5 | ..... | 000100000 | 0x25 | 00100101 |
| 6 | -.... | 000110000 | 0x26 | 00100110 |
| 7 | --... | 000111000 | 0x27 | 00100111 |
| 8 | ---.. | 000111100 | 0x28 | 00101000 |
| 9 | ----. | 000111110 | 0x29 | 00101001 |

## 6.3  Punctuation, Miscellaneous Signs, and Procedural Signals

| Symbol | Name / Role | Morse | SignalCode9 | Token ID | Class |
| --- | --- | --- | --- | --- | --- |
| . | Full stop / period | .-.-.- | 001010101 | 0x30 | 0011 |
| , | Comma | --..-- | 001110011 | 0x31 | 0011 |
| : | Colon / division sign | ---... | 001111000 | 0x32 | 0011 |
| ? | Question mark | ..--.. | 001001100 | 0x33 | 0011 |
| ' | Apostrophe | .----. | 001011110 | 0x34 | 0011 |
| - | Hyphen / dash / subtraction | -....- | 001100001 | 0x35 | 0011 |
| / | Fraction bar / division | -..-. | 000110010 | 0x36 | 0011 |
| ( | Left parenthesis | -.--. | 000110110 | 0x37 | 0011 |
| ) | Right parenthesis | -.--.- | 001101101 | 0x38 | 0011 |
| " | Quotation marks | .-..-. | 001010010 | 0x39 | 0011 |
| = | Double hyphen / equals | -...- | 000110001 | 0x3A | 0100 |
| + | Cross / addition | .-.-. | 000101010 | 0x3B | 0100 |
| @ | Commercial at | .--.-. | 001011010 | 0x3C | 0100 |
| × | Multiplication sign | -..- | 000011001 | 0x3D | 0100 |
| [UNDERSTOOD] | Understood | ...-. | 000100010 | 0x40 | 0101 |
| [ERROR] | Error | ........ | 100000000 | 0x41 | 0101 |
| [INVITE] | Invitation to transmit | -.- | 000001101 | 0x42 | 0101 |
| [WAIT] | Wait | .-... | 000101000 | 0x43 | 0101 |
| [END WORK] | End of work | ...-.- | 001000101 | 0x44 | 0101 |
| [START] | Starting signal | -.-.- | 000110101 | 0x45 | 0101 |

## 6.4  Non-Atomic Source Signs and Expansion

| Source Form | Canonical Handling |
| --- | --- |
| % | No single atomic Morse signal in the ITU procedure. Normalize to 0/0 before atomic encoding. |
| ‰ | Normalize to 0/00 before atomic encoding. |
| ′ | When following figures, transmit the apostrophe signal once. |
| ″ | When following figures, transmit the apostrophe signal twice. |
| Source typography / case | Not part of the Morse signal itself; preserve in a separate text/source model when exact textual round-trip is required. |

ATOMICITY RULE MORSE-32 encodes one atomic transmitted signal per word. A source character that requires multiple Morse signals MUST be normalized into an ordered sequence of atomic MORSE-32 words before projection.

# 7  MORSE-64 Projection Layer

The MORSE-64 extension does not alter the transmitted token. It specifies how the token is projected, performed, observed, or evaluated. The same first 32 bits may therefore be combined with different Projection-32 tails to generate many equivalent perceptual realizations.

## 7.1  UnitDuration10 - B032-B041

UnitDuration10 stores an unsigned scalar from 0 to 1023. The scalar represents the base Morse dot unit. A value of zero means unspecified and causes the renderer to obtain timing from a referenced Timing Profile, environment default, or explicit external Model. Values 1-1023 are interpreted through TimeBasis2.

## 7.2  TimeBasis2 - B042-B043

| Value | Basis |
| --- | --- |
| 00 | Microseconds |
| 01 | Milliseconds |
| 10 | 10 millisecond units |
| 11 | External or higher-resolution time Model |

TimeBasis2 is intentionally compact. Human Morse normally fits comfortably in millisecond or 10 ms resolution, while electronic systems may use microseconds. Scientific work requiring substantially finer or richer timing semantics SHOULD use a higher-resolution descendant such as MORSE-128 or link to a dedicated timing/measurement Kernel rather than overloading MORSE-64.

## 7.3  TimingProfile4 - B044-B047

| Value | Timing Profile |
| --- | --- |
| 0000 | Canonical ITU ratios: dot 1, dash 3, intra-character gap 1, letter gap 3, word gap 7. |
| 0001 | Farnsworth-style spacing; canonical element rhythm with expanded character/word spacing. |
| 0010 | Extended spacing |
| 0011 | Compressed / high-speed registered profile |
| 0100 | Human keying / measured performance |
| 0101 | Machine exact |
| 0110 | Teaching / deliberate |
| 0111 | Accessibility / slow |
| 1000 | Rhythmic / musical |
| 1001 | Custom registered |
| 1010 | Observed reconstruction |
| 1011 | Experimental |
| 1100 | FASK / fictional |
| 1101-1111 | Reserved |

## 7.4  Medium4 - B048-B051

| Value | Projection Medium |
| --- | --- |
| 0000 | Abstract / no renderer |
| 0001 | Audio tone |
| 0010 | RF / carrier keying |
| 0011 | Visible light |
| 0100 | Screen / visual UI |
| 0101 | Haptic / vibration |
| 0110 | Mechanical tap / actuator |
| 0111 | Electrical pulse / data line |
| 1000 | Gesture / movement |
| 1001 | Musical |
| 1010 | Spatial / AR / environment |
| 1011 | Digital logical stream |
| 1100 | Multi-modal |
| 1101 | Custom |
| 1110 | FASK / fictional |
| 1111 | Reserved |

## 7.5  ExpressionMode4 - B052-B055

| Value | Expression Mode |
| --- | --- |
| 0000 | Template |
| 0001 | Generated |
| 0010 | Human keyed |
| 0011 | Human typed |
| 0100 | Observed |
| 0101 | Decoded |
| 0110 | Reconstructed |
| 0111 | Planned |
| 1000 | Playback |
| 1001 | Training target |
| 1010 | Training performance |
| 1011 | Machine received |
| 1100 | Machine sent |
| 1101 | Simulated |
| 1110 | Custom |
| 1111 | Reserved |

## 7.6  Intensity4 - B056-B059

Intensity4 is a normalized 0-15 magnitude. Zero means unspecified or not applicable. Values 1-15 express increasing projection intensity. The renderer interprets magnitude according to medium: audio amplitude, light brightness, haptic strength, motion emphasis, actuator force class, or another registered scale. Absolute engineering quantities SHOULD be stored in a dedicated Model or higher-resolution measurement word when required.

## 7.7  Fidelity4 - B060-B063

Fidelity4 is a normalized 0-15 quality/confidence field. Zero means not evaluated. Depending on ExpressionMode4, Fidelity4 may summarize timing accuracy, decode confidence, received-signal quality, performance similarity, or reconstruction confidence. The governing profile MUST document the interpretation. Fidelity4 does not replace raw measurements or evidence.

# 8  Projection Execution Model

## 8.1  Canonical Timing Derivation

Under TimingProfile4 = 0000, the renderer derives all durations from the base dot unit. ITU-R M.1677-1 defines a dash as three dots, the gap between elements of one character as one dot, the gap between letters as three dots, and the gap between words as seven dots.

Figure 5. Canonical timing model

Given unit U: DOT duration = 1U DASH duration = 3U Within-character gap = 1U Between-letter gap = 3U Between-word gap = 7U

## 8.2  Generic Projection Algorithm

Resolve MORSE-32 Kernel version and Profile4.

Resolve TokenID8 and verify that SignalCode9 matches the registered token under the active profile.

Decode SignalCode9 by locating the sentinel and reading each following bit as DOT=0 or DASH=1.

Resolve UnitDuration10 and TimeBasis2, then apply TimingProfile4.

Resolve Medium4 and target renderer.

Apply Intensity4 through the renderer-specific normalized mapping.

Render each element and gap in order.

If ExpressionMode4 represents an observed or performed signal, evaluate Fidelity4 according to the registered Model.

Emit audit / Bitlink events as configured without modifying MORSE-32.

## 8.3  Medium-Specific Interpretation

| Medium | Projection Behavior |
| --- | --- |
| Audio tone | DOT/DASH become tone-on durations; gaps become silence. |
| RF / carrier | DOT/DASH become keyed-carrier on durations; gaps are carrier-off. |
| Visible light | DOT/DASH become light-on durations; gaps are dark intervals. |
| Haptic | DOT/DASH become vibration pulses; gaps are no-vibration intervals. |
| Mechanical tap | DOT/DASH become actuator contact/hold patterns. |
| Screen / UI | DOT/DASH may animate glyphs, bars, tree traversal, or timed highlights. |
| Gesture / movement | DOT/DASH map to registered short/long movement primitives. |
| Musical | DOT/DASH map to registered short/long musical articulations while preserving Morse timing semantics. |
| Multi-modal | Several renderers subscribe to the same semantic event in synchronization. |

## 8.4  Projection Is Not Alternate Truth

SEPARATION PRINCIPLE MORSE-32 says what signal the token is. MORSE-64 says how that signal is projected or performed in context. Audio samples, light states, vibration states, and UI animations are derivative outputs and MUST NOT silently become independent copies of the Morse semantic truth.

# 9  Semantic Kernel Bitlinking

MORSE-64 is designed to participate directly in CERBERUS Semantic Kernel Bitlinking. Stable fields and values become semantic addresses that can drive renderers, tutorials, interfaces, automations, other Kernels, or logged learning feedback.

## 9.1  Bitlink Scope by Resolution

| Address Scope | Meaning |
| --- | --- |
| MORSE-32 address | Applies to the signal itself and is inherited by MORSE-64 and every conforming descendant. |
| MORSE-64 address | Applies to projection/performance semantics and is inherited only by descendants that preserve the MORSE-64 prefix. |
| Higher-resolution address | Applies only where the descendant Kernel declares and preserves that extension. |

## 9.2  Per-Element Bitlinking

A renderer may treat the decoded SignalCode9 path as an ordered sequence of semantic elements. The first element, second element, and subsequent elements can trigger short/long audio, light, haptic, movement, or teaching outputs without creating a separate stored state for each modality.

Figure 6. Element-level Bitlink projection

MORSE-32: A SignalCode9 -> .- Element 1 DOT x UnitDuration x TimingProfile x Medium x Intensity -> short projection event Element 2 DASH x UnitDuration x TimingProfile x Medium x Intensity -> long projection event

## 9.3  Inherited Bitlinks

A Bitlink registered against a MORSE-32 field does not need to be recreated for MORSE-64. For example, a teaching interface bound to TokenID8 = A or to SignalCode9 = .- can consume the first 32 bits of a MORSE-64 token exactly as if it received MORSE-32. The appended Projection Layer may then add medium-specific behavior without changing the original binding.

## 9.4  Cross-Kernel Translation

Cross-Kernel writes SHOULD follow the Semantic Kernel Bitlinking rule that target mutation is proposed through the target Kernel and validated by the target Rules. A MORSE token may trigger a MUSIC, LIGHTING, HAPTIC, SKENE, UI, or game/fantasy Kernel, but semantic equivalence MUST NOT be assumed merely because a renderer happens to use similar values.

# 10  Semantic Prefix Invariance and Resolution Expansion

## 10.1  Prefix Invariance Law

NORMATIVE LAW Every conforming MORSE-64 word MUST preserve its MORSE-32 prefix byte-for-byte at B000-B031. A future descendant that declares MORSE-64 inheritance MUST preserve B000-B063 byte-for-byte. Expansion occurs by ordered append, not migration.

Figure 7. Append-only resolution growth

MORSE-32 [B000 ---------------- B031] MORSE-64 [B000 ---------------- B031][B032 ---------------- B063] unchanged appended MORSE-128 (illustrative) [B000 ---------------- B031][B032 ---------------- B063][B064 ---------------- B127] unchanged unchanged new resolution

## 10.2  Minimum Sufficient Resolution

| Resolution | Use When |
| --- | --- |
| MORSE-32 | Typing, text conversion, compact transmission tokens, ordinary decode, symbol lookup. |
| MORSE-64 | Audio/light/haptic projection, human keying, machine sending, training, playback, compact performance state. |
| MORSE-128 or linked measurement data | Scientific signal analysis, detailed per-element deviations, carrier/envelope metrics, noise/jitter, richer confidence/evidence. |

A system MUST NOT instantiate higher resolution merely because it exists. The record should carry the lowest resolution sufficient for the operation while higher-resolution Models remain available for interpretation or analysis.

## 10.3  Zero-Loss Upward Extension and Transfer

A stored MORSE-32 token can become MORSE-64 by appending 32 projection bits. The original four bytes are not translated. A MORSE-64-aware system can transfer the full eight bytes directly to another conforming implementation. A MORSE-32-only reader can consume the first four bytes and ignore or preserve the opaque tail. Downward truncation preserves the prefix but omits extension information; it is therefore a lower-resolution projection, not a lossless representation of the larger word unless the tail is retained elsewhere.

# 11  Worked Examples

## 11.1  Example A - Typed Letter A

Figure 8. MORSE-32 encoding of A

Source semantic token: A Morse signal: .- SignalCode9: 000000101 TokenID8: 00000001 (0x01) SymbolClass4: 0001 Letter Role3: 000 Ordinary Profile4: 0000 ITU Version4: 0001 v1 MORSE-32: 000000101 00000001 0001 000 0000 0001

## 11.2  Example B - Same Signal, Different Semantics

Figure 9. Semantic disambiguation

LETTER K SignalCode9: 000001101 MORSE-32: 000001101 00001011 0001 000 0000 0001 INVITATION TO TRANSMIT SignalCode9: 000001101 MORSE-32: 000001101 01000010 0101 000 0000 0001 Physical pattern is identical (-.-). Token identity and class are not.

## 11.3  Example C - SOS as Three Atomic Tokens

Figure 10. SOS as a MORSE-32 sequence

S ... -> MORSE-32 token 1 O --- -> MORSE-32 token 2 S ... -> MORSE-32 token 3 The message is an ordered sequence of atomic words. TimingProfile and stream boundaries determine the gaps between them.

## 11.4  Example D - Haptic Projection of A

Take the MORSE-32 A word unchanged. Append a Projection-32 tail with UnitDuration10 = 80, TimeBasis2 = milliseconds, TimingProfile4 = canonical, Medium4 = haptic, ExpressionMode4 = generated, Intensity4 = 10, Fidelity4 = not evaluated.

Figure 11. MORSE-64 haptic projection

MORSE-32 prefix: 000000101 00000001 0001 000 0000 0001 Projection-32 tail: 0001010000 01 0000 0101 0001 1010 0000 Derived haptic events: DOT -> vibrate 80 ms GAP -> off 80 ms DASH -> vibrate 240 ms

## 11.5  Example E - Human Keying and Fidelity

A training system may retain the target MORSE-32 token while marking ExpressionMode4 = Training Performance and TimingProfile4 = Human Keying / Measured. Detailed observed durations may live in an external measurement record; a registered Model can compare those durations against the target profile and store a compact Fidelity4 result in MORSE-64.

# 12  Runtime, Serialization, and Storage

## 12.1  Canonical Serialization

Bit addresses are numbered B000 through B063 in this specification.

Within each multi-bit field, the specification displays values most-significant-bit first.

A packed MORSE-64 value is exactly 8 bytes. A packed MORSE-32 value is exactly 4 bytes.

Storage implementations MAY expose individual fields for transparency or pack the word natively, provided field boundaries and meanings remain identical.

Textual debug form SHOULD group the word by field boundaries rather than printing an unbroken 64-character string.

## 12.2  Reference Decode Procedure

Figure 12. Reference decoding model

decodeMorse32(word): validate Version4 and Profile4 read SignalCode9 find first 1 sentinel for each following bit: 0 -> DOT 1 -> DASH resolve TokenID8 + SymbolClass4 verify token registry agrees with SignalCode9 return typed atomic signal decodeMorse64(word): core = decodeMorse32(B000-B031) projection = decode fields B032-B063 return core + projection contract

## 12.3  Storage Transparency

An Airtable or educational prototype may store SignalCode9, TokenID8, and the Projection fields as visible columns while also calculating the packed word. A production implementation may store the entire word as 4 or 8 bytes. Both are conforming if the semantic addresses, values, and serialization are equivalent.

# 13  Conformance, Validation, and Testing

## 13.1  MORSE-32 Conformance

Every atomic token has exactly one registered TokenID8 within a profile/version.

Every canonical atomic token has a SignalCode9 that decodes to the declared Morse sequence.

SignalCode9 000000000 is not used as an ordinary atomic signal.

The sentinel must be present and the remaining path must be no longer than eight Morse elements in MORSE-32 v1.

TokenID8, SymbolClass4, and SignalCode9 must agree with the active Profile4 Registry entry.

Ambiguous physical sequences remain semantically distinguishable by token identity/class rather than being collapsed.

## 13.2  MORSE-64 Conformance

B000-B031 match a conforming MORSE-32 word exactly.

Projection fields are decoded according to the declared values and do not modify the MORSE-32 prefix.

TimingProfile4 defines deterministic timing behavior or explicitly references a registered Model.

Medium4 selects a renderer class; renderer-specific absolute units are documented externally where needed.

Intensity4 and Fidelity4 are interpreted according to registered Models and are not silently treated as universal physical units.

Observed, simulated, generated, and training states remain distinguishable through ExpressionMode4 and provenance.

## 13.3  Test Vectors

| Token | Morse | Expected SignalCode9 | Test |
| --- | --- | --- | --- |
| E | . | 000000010 | PASS if decode reproduces exact sequence |
| T | - | 000000011 | PASS if decode reproduces exact sequence |
| A | .- | 000000101 | PASS if decode reproduces exact sequence |
| S | ... | 000001000 | PASS if decode reproduces exact sequence |
| O | --- | 000001111 | PASS if decode reproduces exact sequence |
| 0 | ----- | 000111111 | PASS if decode reproduces exact sequence |
| 5 | ..... | 000100000 | PASS if decode reproduces exact sequence |
| Error | ........ | 100000000 | PASS if decode reproduces exact sequence |

# 14  Governance, Profiles, and Extensions

## 14.1  Canonical ITU Profile

Profile4 = 0000 is reserved for the International Morse mappings aligned to ITU-R M.1677-1. Community or experimental additions MUST NOT silently appear inside the canonical profile. If a new mapping is required, use a registered extension profile, a new Kernel version where appropriate, or a separate descendant Kernel.

## 14.2  Reserved Values

Reserved field values are architectural capacity. Implementations MUST NOT assign private meanings to reserved values and still claim canonical conformance. Private experimentation should use the Custom, Experimental, or FASK values explicitly provided for that purpose.

## 14.3  FASK and Fictional Projection

MORSE-64 can participate in fictional or game logic without corrupting the canonical Morse signal. A FASK profile may cause a Morse token to trigger a spell, portal, character animation, or fictional machine behavior through registered Rules and Bitlinks. The underlying MORSE-32 signal remains interpretable as ordinary Morse when the canonical prefix is preserved.

# 15  Implementation Templates

## 15.1  MORSE-32 Token Registry Record

| Field | Example |
| --- | --- |
| Token ID | 0x01 |
| Canonical Name | A |
| Display Symbol | A |
| Signal | .- |
| SignalCode9 | 000000101 |
| Symbol Class | Letter |
| Role | Ordinary |
| Profile | ITU canonical |
| Version | 1 |
| Aliases / Notes | Optional |
| Bitlinks | Registered consumers |

## 15.2  MORSE-64 Projection Record

| Field | Example |
| --- | --- |
| MORSE-32 Prefix | Exact 32-bit token |
| Unit Duration | 80 |
| Time Basis | Milliseconds |
| Timing Profile | Canonical ITU |
| Medium | Haptic |
| Expression Mode | Generated |
| Intensity | 10/15 |
| Fidelity | 0 = not evaluated |
| Renderer Resource | Haptic profile / device binding |
| Provenance | Source action/event |
| TIME-128 | Optional event coordinate |

# 16  Applications

| Application | Use |
| --- | --- |
| Text and messaging | Compact translation of typed characters into deterministic Morse atomic tokens. |
| Radio and signaling | Projection into keyed carrier or tone timing while keeping the signal identity stable. |
| Accessibility | Haptic, visual, audio, or multimodal Morse from the same MORSE-32 prefix. |
| Education | Teach symbol, dot/dash path, binary tree traversal, timing, and Bitlink behavior from one semantic atom. |
| Human performance | Compare target and keyed/observed signals while preserving the intended token. |
| UI and Skene | Animate tree traversal, character tapping, glyphs, lights, or scene actions from stable semantic triggers. |
| Games and FASK | Use real Morse as a stable input language while attaching fictional Rules and outputs in explicit profiles/worlds. |
| Automation | Map Morse tokens or sequences to registered commands without relying on free-form text parsing. |
| Research extensions | Append higher-resolution fields for signal envelope, jitter, noise, carrier, or measurement detail when required. |

# 17  Design Principles Summary

| Principle | Result |
| --- | --- |
| MORSE-32 = transmitted signal | First 32 bits describe the atomic signal and semantic token. |
| MORSE-64 = projection | Next 32 bits describe how the signal is expressed or evaluated. |
| Dot = 0, Dash = 1 | Natural binary path after a sentinel 1. |
| Token ID disambiguates | Identical signal patterns may carry different semantic purposes. |
| Timing is derived | Store the base unit and timing profile instead of redundant durations. |
| Modality is replaceable | Audio, light, haptic, UI, motion, and other renderers subscribe to one signal. |
| Prefix never migrates | Higher resolution appends fields in order. |
| Use only needed resolution | Do not carry scientific detail for ordinary text conversion. |
| Bitlinks inherit | Bindings to MORSE-32 remain valid inside MORSE-64 descendants. |
| Profiles isolate divergence | Historical, custom, community, experimental, and fictional variants remain explicit. |

# 18  Conclusion

MORSE-64 converts Morse from a display convention into a compact, typed, prefix-invariant Semantic Kernel family. MORSE-32 provides a four-byte atomic transmitted-signal record whose SignalCode9 is itself a binary address through the Morse tree. Token identity and class preserve semantic distinctions that the physical pattern alone cannot express. MORSE-64 then appends four more bytes that define timing, medium, expression mode, intensity, and fidelity without changing the signal.

This separation makes one Morse token reusable across radio, audio, light, haptic, mechanical, digital, spatial, educational, simulation, and fictional systems. The same Bitlinks can remain attached to the stable core while higher-resolution projection or measurement layers are added only when needed. The architecture therefore achieves compact storage, deterministic decode, multimodal portability, transparent learning, and zero-migration semantic growth.

FORMAL SUMMARY MORSE-64 = prefix-invariant MORSE-32 transmitted signal + registered Projection-32 timing/performance semantics + deterministic Bitlink-compatible rendering.

# Appendix A - Complete Bit List

This appendix repeats the canonical B000-B063 allocation as a compact implementation reference.

| Bits | Group | Width | Definition |
| --- | --- | --- | --- |
| B000-B008 | SignalCode9 | 9 | Sentinel-prefixed Morse path, DOT=0, DASH=1. |
| B009-B016 | TokenID8 | 8 | Registered semantic token identity. |
| B017-B020 | SymbolClass4 | 4 | Token class. |
| B021-B023 | Role3 | 3 | Stream/control role. |
| B024-B027 | Profile4 | 4 | Morse profile. |
| B028-B031 | Version4 | 4 | MORSE-32 version. |
| B032-B041 | UnitDuration10 | 10 | Base dot unit scalar. |
| B042-B043 | TimeBasis2 | 2 | Time basis. |
| B044-B047 | TimingProfile4 | 4 | Timing grammar. |
| B048-B051 | Medium4 | 4 | Projection medium. |
| B052-B055 | ExpressionMode4 | 4 | Performance/provenance class. |
| B056-B059 | Intensity4 | 4 | Normalized projection magnitude. |
| B060-B063 | Fidelity4 | 4 | Normalized quality/confidence. |

# Appendix B - Binary Examples

| Token | Morse | SignalCode9 | MORSE-32 grouped binary |
| --- | --- | --- | --- |
| A | .- | 000000101 | 000000101 00000001 0001 000 0000 0001 |
| K | -.- | 000001101 | 000001101 00001011 0001 000 0000 0001 |
| 0 | ----- | 000111111 | 000111111 00100000 0010 000 0000 0001 |
| 5 | ..... | 000100000 | 000100000 00100101 0010 000 0000 0001 |
| Period | .-.-.- | 001010101 | 001010101 00110000 0011 000 0000 0001 |
| Error | ........ | 100000000 | 100000000 01000001 0101 000 0000 0001 |

# Appendix C - Registered Values and Reserved Ranges

| Field | Registry / Reserved Policy |
| --- | --- |
| Token ID | 0x00 reserved/null; 0x01-0x1B letters; 0x20-0x29 figures; 0x30-0x3D punctuation/misc.; 0x40-0x45 procedures; remaining space reserved/extendable by governed registry. |
| SymbolClass4 | 1100-1111 reserved. |
| Role3 | 110 reserved; 111 custom registered. |
| Profile4 | 1000-1111 reserved in v1 except where later governance assigns explicit values. |
| TimingProfile4 | 1101-1111 reserved. |
| Medium4 | 1111 reserved. |
| ExpressionMode4 | 1111 reserved. |
| SignalCode9 | 000000000 reserved for no atomic signal; codes with no registered token remain invalid under the canonical profile. |

# Glossary

| Term | Definition |
| --- | --- |
| Atomic signal | One indivisible Morse token represented by one MORSE-32 word. |
| Bitlink | Registered semantic connection from a Kernel field/state/event to another consumer. |
| Canonical profile | Profile4 = 0000 using mappings aligned to ITU-R M.1677-1. |
| Composite source sign | Source symbol that must be expanded into multiple atomic Morse signals before transmission. |
| Dash | Long Morse element; encoded as binary 1 after the SignalCode9 sentinel. |
| Dot | Short Morse element; encoded as binary 0 after the SignalCode9 sentinel. |
| ExpressionMode4 | Four-bit class describing how the projection was produced or observed. |
| FASK | Fantastically Altered Semantic Kernel / explicitly fictional or alternate-world semantic extension. |
| Fidelity4 | Four-bit normalized quality or confidence indicator. |
| Intensity4 | Four-bit normalized projection magnitude. |
| Medium4 | Four-bit renderer/medium class. |
| Minimum Sufficient Resolution | Use the smallest word that carries the semantic depth required by the task. |
| MORSE-32 | 32-bit transmitted-signal semantic core. |
| MORSE-64 | 64-bit word containing MORSE-32 plus Projection-32. |
| Prefix Invariance | Published lower-resolution bits retain the same addresses and meanings in descendants. |
| Profile4 | Four-bit Morse mapping/profile selector in MORSE-32. |
| Projection | Replaceable physical or perceptual realization of a semantic Morse signal. |
| Role3 | Three-bit stream/control role. |
| Semantic Kernel | Versioned fixed-width semantic contract assigning stable meaning to bit addresses and groups. |
| Sentinel | Leading 1 placed before the dot/dash path in SignalCode9. |
| SignalCode9 | Nine-bit exact atomic Morse tree-path encoding. |
| SymbolClass4 | Four-bit semantic token class. |
| TimeBasis2 | Two-bit basis for interpreting UnitDuration10. |
| TimingProfile4 | Four-bit projection timing grammar selector. |
| TokenID8 | Eight-bit registered semantic identity. |
| UnitDuration10 | Ten-bit scalar defining the base dot-time unit. |
| Version4 | Four-bit MORSE-32 Kernel version field. |

# Subject Index

| Subject | Primary Sections |
| --- | --- |
| Ambiguous signals | 5.3; 11.2 |
| Append-only extension | 10.1-10.3 |
| Audio | 7.4; 8.3; 16 |
| Binary tree | 5.1-5.2 |
| Bitlinking | 9 |
| Canonical timing | 7.3; 8.1 |
| Composite signs | 6.4 |
| Conformance | 13 |
| Dash | 5.1; Glossary |
| Dot | 5.1; Glossary |
| Expression mode | 7.5 |
| FASK | 14.3; 16 |
| Fidelity | 7.7 |
| Figures | 6.2 |
| Haptic | 7.4; 11.4; 16 |
| Intensity | 7.6 |
| ITU profile | 6; 14.1; References |
| Letters | 6.1 |
| Medium | 7.4; 8.3 |
| Minimum Sufficient Resolution | 10.2 |
| MORSE-32 | 4.1; 5; 10 |
| MORSE-64 | 4.2; 7-10 |
| MORSE-128 | 10.2 |
| Prefix Invariance | 10 |
| Procedural signals | 6.3 |
| Projection | 7-8 |
| Punctuation | 6.3 |
| Role | 5.5 |
| Serialization | 12 |
| SignalCode9 | 5.1-5.2 |
| Symbol Class | 5.4 |
| Timing Profile | 7.3 |
| Token ID | 5.3; 6 |
| Training | 7.5; 11.5; 16 |
| Unit Duration | 7.1-7.2 |
| Versioning | 5.7; 10; 14 |

# References

ITU-R M.1677-1 (10/2009), International Morse code. International Telecommunication Union, Radiocommunication Sector. Status: In force.

CERBERUS Semantic Kernel Bitlinking - Formal Specification for Bit-Addressed Operators, Rule-Governed State Transitions, Cross-Kernel Translation, and Multi-Modal Semantic Triggers, v1.0.

Semantic Prefix Invariance - Formal System Specification, v1.0.

CERBERUS Matryoshka Integrated Formal Specification, v1.0.

SOURCE NOTE Canonical Morse character mappings and the 1:3:1:3:7 timing relationships in this specification are aligned to ITU-R M.1677-1. MORSE-32 and MORSE-64 field layouts, Token IDs, profiles, projection semantics, Bitlink behavior, and prefix-invariance rules are CERBERUS architecture defined by this specification.